# Intelligent AI Assistant Integration

## Implementation Report — Checkpoint 3

### Task 3.1 — Research & Integrate AI Assistant SDK

## 1. Overview and Objective

This section documents the implementation of the AI assistant for the All-in-one Home Smart Box. The assistant provides users with a natural language interface to query system status, control services, and ask open-ended questions — all processed locally on the Raspberry Pi 5 without reliance on cloud services.

This work directly addresses Objective 3, Task 3.1 of the project: “Research and select the final open model platform. Utilize the Hailo SDK to convert and optimize a lightweight LLM for efficient execution on the Hailo-8 NPU.”

A critical finding during implementation was that the Hailo-8 NPU is purpose-built for CNN inference (vision models) and cannot run large language models. LLMs require general-purpose matrix operations that the NPU’s fixed-function pipeline does not support. Consequently, the architecture was revised: the LLM runs on the Pi 5’s quad-core Cortex-A76 CPU via Ollama, while the Hailo-8 remains dedicated to the vision pipeline (Task 3.2). This separation is documented as a deviation from the progress report and represents a more informed hardware-workload partitioning strategy.

## 2. System Architecture

The AI assistant uses a hybrid two-tier architecture that combines a fast pattern-based intent matcher with an LLM fallback, connected to the unified interface via both MQTT and an HTTP API.

| Component | Technology | Role |
| --- | --- | --- |
| Intent Matcher | Python regex patterns | Instant response (~0.5s) for known command categories (CPU temp, RAM, disk, containers, network, camera status). No LLM invocation required. |
| LLM Engine | Ollama (Docker) + qwen2.5:1.5b | Handles open-ended queries that don’t match any intent pattern. Produces contextual responses using system state as prompt context. |
| Message Bus | MQTT (Mosquitto) | Receives queries on smartbox/assistant/query, publishes responses to smartbox/assistant/response. Enables integration with Home Assistant automations. |
| HTTP API | Python HTTPServer (port 8087) | RESTful endpoint (/ask?q=...) enabling the kiosk dashboard to send queries via JavaScript fetch() calls. |
| Resource Manager | systemd stop/start | Temporarily pauses the detection service during LLM inference to free CPU resources, then automatically resumes it after the response is generated. |

Data Flow — Intent Match (fast path): User Query → Intent Matcher → Pattern Match → Execute System Command → Return Response (~0.5s)

Data Flow — LLM Fallback (slow path): User Query → No Pattern Match → Pause Detection Service → Query Ollama → Generate Response → Resume Detection Service → Return Response (~5-12s)

## 3. LLM Model Selection and Benchmarking

### 3.1 Hardware Constraints

The Raspberry Pi 5 provides a quad-core ARM Cortex-A76 CPU at 2.4 GHz with 16 GB LPDDR4X RAM. While sufficient for running quantized small language models, the CPU must also serve the operating system, Docker containers (Home Assistant, Jellyfin, OMV, Mosquitto), the kiosk display server, and the Hailo detection pipeline. This multi-service workload significantly constrains the resources available for LLM inference.

### 3.2 Initial Model: llama3.2:3b

The initial model evaluated was Meta’s Llama 3.2 at the 3B parameter size, a popular open-source model with strong general capabilities. Ollama was deployed as a Docker container with model storage on the 413 GB NVMe data partition:

sudo docker run -d \
  --name ollama \
  --restart=always \
  -p 11434:11434 \
  -v /srv/data/ollama:/root/.ollama \    # Models stored on large partition
  ollama/ollama

Baseline benchmarks with the detection service stopped (CPU fully available):

Model: llama3.2:3b
Prompt eval rate: 23.52 tokens/s
Eval (generation) rate: 5.19 tokens/s
Total response time: ~13.3s for a one-sentence answer

### 3.3 Alternative Model: qwen2.5:1.5b

To improve response times, the Alibaba Qwen 2.5 model at 1.5B parameters was evaluated. At roughly half the parameter count, it requires less computation per token while maintaining acceptable quality for the assistant’s use case (system queries and short informational responses):

Model: qwen2.5:1.5b
Prompt eval rate: 63.00 tokens/s
Eval (generation) rate: 13.82 tokens/s
Total response time: ~1.0s for a short answer (detection stopped)

### 3.4 Comparative Analysis

| Metric | llama3.2:3b | qwen2.5:1.5b |
| --- | --- | --- |
| Parameters | 3 billion | 1.5 billion |
| Model Size (disk) | 2.0 GB | 986 MB |
| Prompt Eval Rate (no detection) | 23.52 tokens/s | 63.00 tokens/s |
| Generation Rate (no detection) | 5.19 tokens/s | 13.82 tokens/s |
| Typical Response Time (no detection) | ~13s | ~5s |
| Response Quality | Higher fluency, longer responses | Adequate for system queries |
| RAM Usage | ~3.1 GB | ~1.15 GB |

Decision: qwen2.5:1.5b was selected as the primary model for the assistant due to its significantly faster inference speed (2.6× faster generation) and lower memory footprint (63% less RAM). For the assistant’s primary use case — answering system status queries and short informational questions — the quality difference is negligible, while the speed improvement directly impacts user experience.

## 4. CPU Resource Contention Analysis

### 4.1 The Problem

When the detection service (Task 3.2) runs simultaneously with Ollama, LLM performance degrades catastrophically:

| Condition | qwen2.5:1.5b Generation Rate |
| --- | --- |
| Detection stopped (CPU idle) | 13.82 tokens/s |
| Detection at 10 FPS | 0.72 tokens/s |
| Detection at 5 FPS | 0.38 tokens/s |
| Detection at 5 FPS, no –use-frame | 0.38 tokens/s |
| Detection stopped, CPU pinned to cores 2-3 | 0.32 tokens/s |

The detection service consumes nearly 100% of all four CPU cores due to the GStreamer pipeline overhead (camera capture, ISP processing, frame routing, YOLO pre/post-processing). Even reducing the frame rate from 10 to 5 FPS had negligible impact because the pipeline’s base overhead is constant regardless of frame rate.

### 4.2 Root Cause Investigation

Using htop, the following CPU consumption breakdown was identified:

- picamera2: Camera capture and Image Signal Processor (ISP) conversion from raw Bayer sensor data to RGB — runs continuously regardless of frame rate

- GStreamer pipeline: Frame routing, buffering, and format conversion between pipeline stages — multiple threads with constant overhead

- YOLO pre-processing: Frame resizing to model input dimensions before NPU transfer

- NMS post-processing: Non-Maximum Suppression runs on CPU via the Hailo SDK’s .so library

- Callback overhead: Bounding box drawing (OpenCV), JPEG encoding for MJPEG stream, MQTT publishing

While the neural network inference itself runs on the Hailo-8 NPU, everything around it runs on the CPU, creating a pipeline that saturates all four cores.

### 4.3 Solutions Evaluated

| Approach | Result | Outcome |
| --- | --- | --- |
| Reduce frame rate (10 → 5 FPS) | 0.72 → 0.38 tokens/s | Failed — pipeline overhead is constant |
| Remove –use-frame (no MJPEG) | 0.38 tokens/s (unchanged) | Failed — JPEG encoding is not the bottleneck |
| CPU pinning (detection on cores 0-1, Ollama on 2-3) | 0.32 tokens/s | Failed — 2 cores insufficient for either service |
| Pause detection during LLM inference | 13.82 tokens/s | Adopted — full CPU available for LLM |

### 4.4 Adopted Solution: Intelligent Resource Scheduling

The hybrid pause/resume architecture was implemented as the production solution. When a query requires the LLM (i.e., no intent pattern matches), the assistant:

- Stops the detection systemd service (systemctl stop detection)

- Waits 1 second for CPU to settle

- Sends the prompt to Ollama

- Receives the complete response

- Restarts the detection service (systemctl start detection)

This approach provides the full 13.82 tokens/s generation rate while the LLM is active, with detection automatically resuming after the response completes. The typical LLM query takes 5-12 seconds, during which camera monitoring is briefly paused.

A passwordless sudoers rule was created to allow the pi user to stop and start the detection service without authentication prompts:

# /etc/sudoers.d/assistant
pi ALL=(ALL) NOPASSWD: /usr/bin/systemctl stop detection
pi ALL=(ALL) NOPASSWD: /usr/bin/systemctl start detection

This resource scheduling approach is an engineering trade-off: it sacrifices continuous camera monitoring for usable LLM response times. For a home environment, a 5-12 second gap in detection coverage during an AI query is acceptable, whereas a 60+ second response time (or timeout failure) renders the LLM unusable.

## 5. Intent Matcher Implementation

### 5.1 Design Rationale

The intent matcher was developed to handle the most common user queries without invoking the LLM at all. This provides two benefits: instant responses (~0.5s including the system data fetch) and no interruption to the detection service.

The matcher uses compiled regular expression patterns grouped by intent category:

INTENT_PATTERNS = {
    "cpu_temp": [
        r"cpu\s*(temp|temperature)",
        r"how\s*hot",
        r"temperature",
    ],
    "cpu_usage": [
        r"cpu\s*(usage|load|percent)",
        r"how\s*(busy|loaded)\s*(is)?\s*(the)?\s*(cpu|processor)",
    ],
    "ram_usage": [
        r"(ram|memory)\s*(usage|free|available|used)",
        r"how\s*much\s*(ram|memory)",
    ],
    "disk_usage": [
        r"(disk|storage)\s*(usage|space|free|available)",
    ],
    "uptime": [
        r"uptime",
        r"how\s*long.*running",
    ],
    "system_status": [
        r"(system|box)\s*status",
        r"status\s*report",
    ],
    "containers": [
        r"(docker\s*)?(container|service)s?\s*(status|running)?",
        r"what\s*(services?|containers?)\s*(are)?\s*running",
    ],
    "network": [
        r"(ip|network)\s*(address|info)",
        r"what\s*(is|s)\s*(my|the)\s*ip",
    ],
    "camera_status": [
        r"(camera|detection)\s*(status|running)",
        r"is\s*(the)?\s*camera\s*(on|running)",
        r"anyone\s*(at|near)\s*(the)?\s*(door|home)",
    ],
    "help": [
        r"^help$",
        r"what\s*can\s*you\s*do",
    ],
}

### 5.2 Data Sources

Each intent handler fetches real-time data from the Pi’s system interfaces:

| Intent | Data Source | Method |
| --- | --- | --- |
| CPU temperature | /stats API (pi-stats.py) | Reads /sys/class/thermal/thermal_zone0/temp |
| CPU usage | /stats API | Two-sample /proc/stat delta |
| RAM usage | /stats API | Parses /proc/meminfo |
| Disk usage | /stats API | os.statvfs('/') |
| Uptime | /stats API | Reads /proc/uptime |
| Network IP | /stats API | UDP socket method |
| Container status | docker ps | Subprocess command execution |
| Camera status | systemctl is-active detection | Subprocess command execution |

### 5.3 System Status Report

The system_status intent aggregates multiple metrics into a single formatted response:

def handle_intent(intent):
    stats = get_system_stats()          # Single API call to /stats

    if intent == "system_status":
        return (
            f"System Status Report:\n"
            f"  CPU: {stats['cpu_percent']}% at {stats['cpu_temp']}°C\n"
            f"  RAM: {stats['ram_percent']}% used\n"
            f"  Disk: {stats['disk_percent']}% used\n"
            f"  Uptime: {stats['uptime']}\n"
            f"  IP: {stats['ip']}"
        )

Sample output:

System Status Report:
  CPU: 33.3% at 46.3°C
  RAM: 19.1% used
  Disk: 38.6% used
  Uptime: 1d 0h 32m
  IP: 192.168.31.117

## 6. LLM Integration via Ollama

### 6.1 Ollama API Integration

When no intent pattern matches, the query is forwarded to Ollama’s REST API. The LLM is given a system prompt that contextualises it as the Smart Box assistant, and current system metrics are injected into the prompt for context-aware responses:

def query_ollama(prompt):
    system_prompt = (
        "You are the AI assistant for a Raspberry Pi 5-based smart home "
        "device called the 'Home Smart Box'. You help users with their "
        "smart home, media, and system questions. Keep responses concise "
        "(2-3 sentences max). You have access to Home Assistant, Jellyfin "
        "media server, OpenMediaVault NAS, and an AI camera."
    )

    # Inject live system state into the prompt
    stats = get_system_stats()
    context = (
        f"Current system state: CPU={stats['cpu_percent']}%, "
        f"Temp={stats['cpu_temp']}°C, RAM={stats['ram_percent']}%, "
        f"Disk={stats['disk_percent']}%"
    )

    response = requests.post(
        f"{OLLAMA_URL}/api/generate",
        json={
            "model": OLLAMA_MODEL,
            "prompt": f"{context}\n\nUser: {prompt}",
            "system": system_prompt,
            "stream": False,
            "options": {
                "num_predict": 100,    # Limit response length for speed
                "temperature": 0.7,
            }
        },
        timeout=120
    )
    return response.json().get("response", "Sorry, I couldn't generate a response.")

Key design decisions in the Ollama integration:

- num_predict: 100: Caps the maximum generated tokens to prevent excessively long responses that would keep the detection service paused for too long.

- temperature: 0.7: Balances creativity with factual accuracy for system-related queries.

- stream: False: Waits for the complete response before returning, simplifying the HTTP API response handling.

- timeout: 120: Extended timeout to accommodate the model loading time on first query after Ollama restart.

### 6.2 Context-Aware Responses

By injecting the current system state into every LLM prompt, the assistant can reference actual system metrics in its responses. For example, when asked “how is the system doing?”, the LLM can respond with specific numbers rather than generic advice, even though this query falls through to the LLM path rather than the intent matcher.

## 7. Dashboard Chat Interface

### 7.1 HTTP API Server

An HTTP API server was embedded within the assistant service as a background thread, running on port 8087 (port 8086 was occupied by InfluxDB). The API provides a simple GET endpoint:

GET /ask?q=your+question

Response format:

{
    "query": "what is the cpu temperature",
    "response": "The CPU temperature is 46.3°C.",
    "method": "intent_match",
    "elapsed_seconds": 0.5
}

The server includes CORS headers (Access-Control-Allow-Origin: *) to allow cross-origin requests from the dashboard, and an OPTIONS handler for CORS preflight requests.

### 7.2 Nginx Reverse Proxy

The HTTP API was proxied through the existing Nginx dashboard server to avoid exposing an additional port and to enable same-origin requests from the dashboard JavaScript:

location /ask {
    proxy_pass http://127.0.0.1:8087/ask;
    proxy_set_header Host $host;
}

This allows the dashboard to call /ask?q=... as a relative URL, which is then forwarded to the assistant’s HTTP server on port 8087.

### 7.3 Dashboard Chat Widget

A chat interface was added to the kiosk dashboard (/var/www/dashboard/index.html) consisting of a scrollable message area and a text input with an “Ask” button. The JavaScript sends queries via fetch() and displays responses inline with timing metadata:

function sendChat() {
    const query = input.value.trim();
    if (!query) return;

    // Display user message immediately
    messages.innerHTML += '<div style="color:var(--accent)">You: ' + query + '</div>';
    messages.innerHTML += '<div id="botTyping">Smart Box: thinking...</div>';

    // Send to assistant API via Nginx proxy
    fetch('/ask?q=' + encodeURIComponent(query))
      .then(r => r.json())
      .then(data => {
        // Replace "thinking..." with actual response
        typing.innerHTML = 'Smart Box: ' + data.response +
          '<span>(' + data.elapsed_seconds + 's via ' + data.method + ')</span>';
      });
}

The chat widget displays the response method (intent_match or llm) and elapsed time alongside each response, providing transparency about whether the fast path or LLM fallback was used.

## 8. Technical Challenges and Solutions

### 8.1 Hailo-8 Cannot Run LLMs

Problem: The progress report proposed running the LLM on the Hailo-8 NPU. During implementation, it was determined that the Hailo-8 is a fixed-function CNN accelerator that cannot execute the transformer architecture operations required by large language models.

Root Cause: The Hailo-8’s dataflow architecture is optimised for convolutional and pooling operations with fixed tensor shapes. LLM inference requires dynamic sequence-length attention mechanisms, key-value caching, and autoregressive token generation — operations that the Hailo-8’s hardware pipeline does not support.

Resolution: The LLM was deployed on the CPU via Ollama running in a Docker container. This is documented as a deviation from the progress report. The revised architecture actually provides a cleaner separation of concerns: the NPU handles continuous vision inference while the CPU handles on-demand language inference, with the resource scheduler mediating between them.

### 8.2 Catastrophic LLM Performance Under CPU Contention

Problem: With the detection service running, LLM generation dropped from 13.82 tokens/s to 0.72 tokens/s — a 19× slowdown that rendered the LLM unusable (60+ second timeouts).

Root Cause: The GStreamer/picamera2 detection pipeline consumes nearly 100% of all four CPU cores with constant overhead from camera ISP processing, frame buffering, and YOLO pre/post-processing. This overhead is independent of the configured frame rate.

Resolution: A hybrid intent-match + pause/resume architecture was implemented. Known commands bypass the LLM entirely (0.5s response), while LLM queries temporarily pause the detection service to free all four CPU cores (5-12s response at 13.82 tokens/s), then automatically resume detection.

### 8.3 Docker CPU Pinning Made Performance Worse

Problem: An attempt to isolate workloads by pinning the detection service to CPU cores 0-1 and Ollama to cores 2-3 via taskset and docker update --cpuset-cpus resulted in even worse performance (0.32 tokens/s).

Root Cause: Both services require all four cores to operate efficiently. The detection pipeline’s multi-threaded GStreamer stages cannot be effectively constrained to two cores without starving critical threads, and Ollama’s matrix multiplication kernels benefit from the full 4-core width.

Resolution: CPU pinning was removed. The pause/resume approach was confirmed as the only viable solution given the hardware constraints, as it provides each service with the full 4-core CPU when active.

### 8.4 Ollama Container Retained CPU Pinning After Update

Problem: After running docker update --cpuset-cpus="" ollama to remove CPU pinning, htop still showed Ollama only using 2 cores. The docker update command did not take effect.

Resolution: The Ollama container was recreated from scratch:

sudo docker stop ollama && sudo docker rm ollama
sudo docker run -d --name ollama --restart=always \
  -p 11434:11434 -v /srv/data/ollama:/root/.ollama ollama/ollama

Since models are stored in the mounted volume (/srv/data/ollama), no data was lost. The new container correctly used all four CPU cores, restoring the expected 13.82 tokens/s generation rate.

### 8.5 Port Conflict with InfluxDB

Problem: The HTTP API server was initially configured on port 8086. On startup, the assistant crashed with OSError: [Errno 98] Address already in use.

Root Cause: InfluxDB (installed as part of the system’s monitoring stack) uses port 8086 as its default HTTP API port. This was discovered using fuser -v 8086/tcp.

Resolution: The assistant’s API port was changed from 8086 to 8087 in both the Python script (API_PORT = 8087) and the Nginx proxy configuration.

### 8.6 Assistant Service Crash Loop Due to Import Error

Problem: The systemd service failed repeatedly with ModuleNotFoundError: No module named 'HTTPServer'.

Root Cause: During editing, import HTTPServer was accidentally added as a standalone import at the top of the file. The correct import is from http.server import HTTPServer, BaseHTTPRequestHandler, which was already present in the HTTP API section of the code.

Resolution: The erroneous standalone import line was removed. The existing from http.server import HTTPServer, BaseHTTPRequestHandler import within the API server section was confirmed as correct.

### 8.7 Dashboard Chat Missing on Kiosk After Container Navigation

Problem: The AI assistant chat widget appeared correctly on the dashboard when viewed from a laptop browser, but disappeared on the Pi’s kiosk display after navigating to a container (e.g., Home Assistant) and returning to the dashboard.

Root Cause: Chromium’s aggressive caching served a stale version of the dashboard HTML from before the chat widget was added.

Resolution: Cache-control headers were added to the Nginx configuration to force Chromium to always fetch the latest version of the dashboard:

location / {
    try_files $uri $uri/ =404;
    add_header Cache-Control "no-cache, no-store, must-revalidate";
    add_header Pragma "no-cache";
    add_header Expires 0;
}

Additionally, Chromium’s cache directories were cleared manually before rebooting:

rm -rf /home/pi/.config/chromium/Default/Cache
rm -rf /home/pi/.config/chromium/Default/Code\ Cache

## 9. Summary of Technical Challenges

| # | Challenge | Root Cause | Resolution |
| --- | --- | --- | --- |
| 1 | Hailo-8 cannot run LLMs | NPU is a fixed-function CNN accelerator | LLM runs on CPU via Ollama; NPU dedicated to vision |
| 2 | LLM at 0.72 tokens/s with detection running | GStreamer pipeline saturates all 4 CPU cores | Hybrid pause/resume: stop detection during LLM inference |
| 3 | CPU pinning worsened performance | Both services need all 4 cores | Removed pinning; adopted pause/resume instead |
| 4 | Docker retained CPU pinning | docker update didn’t apply | Recreated container from scratch; volume preserved models |
| 5 | Port 8086 conflict with InfluxDB | InfluxDB default HTTP port | Changed assistant API to port 8087 |
| 6 | Import error crash loop | Accidental import HTTPServer line | Removed erroneous import |
| 7 | Chat missing on kiosk after navigation | Chromium aggressive caching | Added no-cache headers; cleared browser cache |

## 10. Production Deployment

### 10.1 systemd Service Configuration

[Unit]
Description=Smart Box AI Assistant
After=network.target docker.service detection.service
Wants=docker.service

[Service]
Type=simple
User=pi
Environment=PATH=/home/pi/hailo-apps/venv_hailo_apps/bin:/usr/local/bin:/usr/bin:/bin
Environment=VIRTUAL_ENV=/home/pi/hailo-apps/venv_hailo_apps
WorkingDirectory=/home/pi/hailo-apps
ExecStart=/home/pi/hailo-apps/venv_hailo_apps/bin/python3 /home/pi/hailo-apps/ai_assistant.py
Restart=on-failure
RestartSec=10

[Install]
WantedBy=multi-user.target

### 10.2 File Summary

| File Path | Purpose |
| --- | --- |
| ~/hailo-apps/ai_assistant.py | Main assistant script: intent matcher + Ollama LLM + MQTT + HTTP API |
| /etc/systemd/system/assistant.service | systemd unit for auto-start and crash recovery |
| /etc/sudoers.d/assistant | Passwordless sudo for detection service pause/resume |
| /etc/nginx/sites-available/dashboard | Updated with /ask and /health proxy locations |
| /var/www/dashboard/index.html | Updated with AI assistant chat widget |

## 11. Results and Validation

### 11.1 Performance Results

| Scenario | Method | Response Time | Detection Running |
| --- | --- | --- | --- |
| “what is the cpu temperature” | Intent match | 0.5s | Yes (uninterrupted) |
| “system status” | Intent match | 0.5s | Yes (uninterrupted) |
| “what containers are running” | Intent match | 0.5s | Yes (uninterrupted) |
| “suggest a movie in one sentence” | LLM (qwen2.5:1.5b) | 5.2s | Paused, then resumed |
| “what is best movie in existence?” | LLM (qwen2.5:1.5b) | 9.3s | Paused, then resumed |
| “what do you do?” | LLM (qwen2.5:1.5b) | 12.6s | Paused, then resumed |

### 11.2 Validation Against Task 3.1 Success Criteria

| Success Criterion | Result | Status |
| --- | --- | --- |
| LLM deployed and running locally on the Pi 5 | qwen2.5:1.5b running via Ollama Docker container, models stored on NVMe | Met |
| Assistant capable of processing text queries | Both intent matching and LLM processing functional via MQTT and HTTP API | Met |
| Queries for system status return accurate data | Real-time stats from /proc, /sys, and Docker CLI verified against actual values | Met |
| Response latency under 5 seconds for common queries | Intent-matched queries respond in 0.5s; LLM queries respond in 5-12s | Partially Met |
| Integration with unified user interface | Chat widget on kiosk dashboard + MQTT sensors in Home Assistant | Met |

### 11.3 Evidence of Work

Dashboard Chat Interface — Live Testing:

The following interactions were captured from the kiosk dashboard chat interface:

You: what is the cpu temperature
Smart Box: The CPU temperature is 46.3°C. (0.5s via intent_match)

You: what is best movie in existence?
Smart Box: There's no definitive "best" movie because opinions vary widely
based on personal taste. However, some universally acclaimed films are
The Shawshank Redemption, Schindler's List, Pulp Fiction, and The Godfather.
These films have won numerous awards and are highly regarded for their
storytelling and performances. (9.3s via llm)

You: also what do u do?
Smart Box: As a smart home device, the Home Smart Box automates various tasks
for you without manual intervention. You can set up automation rules to control
lights, locks, and thermostats based on your schedule or presence detection.
Additionally, with media integration, it could stream music from Jellyfin to
speakers in different rooms using an AI camera for video monitoring if needed.
The system also manages your OpenMediaVault NAS, allowing you to securely
upload, download, and share files seamlessly. (12.58s via llm)

MQTT Message Flow — Verified via mosquitto_sub:

{"query": "system status",
 "response": "System Status Report:\n  CPU: 33.3% at 46.3°C\n  RAM: 19.1% used\n  Disk: 38.6% used\n  Uptime: 1d 0h 32m\n  IP: 192.168.31.117",
 "method": "intent_match",
 "model": "intent_matcher",
 "elapsed_seconds": 0.51,
 "timestamp": "2026-04-04T05:13:38.103234",
 "id": "1"}

## 12. Relevance to Project Objectives

This implementation fulfills Objective 3, Task 3.1 of the project. While the original plan to run the LLM on the Hailo-8 NPU was not feasible (the NPU is a CNN accelerator, not a general-purpose AI processor), the revised CPU-based LLM architecture with intelligent resource scheduling demonstrates a deeper understanding of hardware capabilities and workload management than a straightforward NPU deployment would have shown.

The hybrid intent-match + LLM architecture applies concepts from ELEC3300 (Embedded Systems) in real-time system design: the intent matcher provides deterministic, bounded response times for critical system queries (analogous to hard real-time constraints), while the LLM provides best-effort responses for non-critical open-ended queries (analogous to soft real-time). The resource scheduling mechanism (pause/resume detection) applies concepts from ELEC3120 (Computer Communication Networks) regarding resource multiplexing and quality-of-service management on shared infrastructure.

The deviation from the Hailo-8 LLM plan to the CPU-based Ollama architecture is documented as a design decision informed by empirical testing rather than a failure. It demonstrates the engineering practice of adapting designs based on actual hardware behaviour, which is a hallmark of competent embedded systems development.